General Instructions

How to

Replace the Rubber Tire on the

4v-Series Grob Saws

#1 Remove pulley from saw.

#2 Strip all old rubber and glue off of pulley with an ethyl acetate solution.

#3 Make or find a round plug that can fit in a vice and the bore of the pulley.

#4 Place plug vertically in vise and install pulley on plug.

#5 Clean the new tire using Mineral Spirits or paint thinner.

#6 Brush glue into the groove of the pulley, allow to get tacky.

#7 Stretch the new tire onto the pulley but do not let go yet.

#8 Insert rod or screw driver shank, between tire and pulley.

#9 Holding the rod stationary, rotate the pulley about 3 turns to equalize the rubber.

#10 Remove rod, make sure rubber is seated well in corners of groove.

#11 Clean excess glue from pulley.

#12 Let dry for 5 to 6 hours. (Over night if possible)

Use 3m #1357 Rubber Cement or Equivalent

2013