Installing New Saw Band on Grob 6V-36 Band Saw

Turn power off at electrical box.

Open upper and lower doors.

Remove upper saw band guard.

Loosen upper and lower guide holders in the guide bars. Do not remove.

Install new saw band centering, front to back, on each pulley.

Turn upper crank handle located on left side of saw frame clock-wise to put tension on saw band. Tension indicator located on front face of saw frame should read the same size as the saw band being installed.

Move transmission levers to neutral.

Slowly turn pulleys by hand to see where saw band tracks on pulleys.

You want the saw band to track near the center of both pulleys when turned. If not centered, while turning pulleys by hand turn tracking knob located on right side of saw frame to set tracking.

When saw band is tracked correctly slide both upper and lower guide holders forward to just make contact with the saw band. Loosen the set screws holding the side guides in the holders. Slide side guides inward to just make contact with the side of the saw band and tighten set screws. Make sure guide holder is still forward and just making contact with the rear of the saw band and tighten the holder clamp nut. Repeat for other guide holder.

Re-install upper saw band guard, close both doors, re-set brake (if brake was set from saw band breaking) by pushing knob all the way up until you hear and feel latch setting itself, lower knob, turn power back on.